Quick Check :

User for Ethernet-FAM is suggest to perform a quick test to confirm the board is functioning.

- Install the CP2101 USB-Serial bridge driver on the CP2101_Driver Directory on the CD-ROM. The driver MUST BE INSTALLED BEFORE USING THE USB CABLE TO CONNECT TO THE ETHERNET-FAM.

- Connect the USB cable to the Ethernet-FAM, please reference to the “Ethernet_FAM_Connection.DOC”.

- Check control panel and determine which COM port the USB-Serial bridge is using, say COM4

- Run the FAM_demo_ethernet.EXE on the CD :\Test_Software\ directory.

- Adjust the FAM Speed setting to 921600 bps

- Click Start

- Put the finger on the Ethernet-FAM windows and the Fingerprint View will show the image of the fingerprint.

- Please remember to set the FAM Speed back to the speed it needs to be used for connecting other device. Once the FAM Speed is set, it will maintain the speed until another set or power off. The default speed is 115200 bps.

- The Ethernet-FAM is functioning correctly now and please contacts Futronic if there is any question.