FAM DEMO MANUAL

22-FEB-2006

1. Add User

-1. Enter number of samples to be captured.

-2. Check the checkbox of VIP USER if it is VIP user.

-3. Enter the USER ID.

-4. Place the finger on the Scanner and Press the “GetImage” Button.

-5. Press the “Processing” Button.

-6. Press the “Store #” Button.

-7. Repeat 4,5,6 until the “Add” Button becomes active. Press the “Add” Button.

Note: If the program does not work, please check whether COM Port selection is correct.

Please refer to FAM 80.00 serial command description.doc

“GetImage” Button corresponds to “Capture fingerprint image command”(B.2.).

“Processing” Button corresponds to “Process the captured fingerprint image command”(B.3).

“Store #” Button corresponds to “Store the sample in FAM RAM command”(B.7.).

In this example, max number of samples is 3. Then Sample N is from 0 to 2.

“Add” Button corresponds to “Store the fingerprint template to FAM flash memory command”(B.6.).

Please do not send “Processing” command until Error code of “GetImage” is RESULT_OK(0x40).

If Error code of “Processing” is not RESULT_OK, please do “GetImage” again.

2. Recognize VIP user

-1. Check the checkbox of VIP USER.

-2. Place the finger on Scanner and Press the “GetImage” Button.

-3. Press the “Processing” Button.

-4. Press the “Recognize” Button.

-5. Fingerprint view will show the result. (For details, please refer to FAM 90.23 serial command description.doc B.5.)

3. Recognize ordinary user

-1. Enter the USER ID.

-2. Place the finger on Scanner and Press the “GetImage” Button.

-3. Press the “Processing” Button.

-4. Press the “Recognize” Button.

-5. Fingerprint view will show the result. (For details, please refer to FAM 90.23 serial command description.doc B.4.)

4. Delete users.

-1. Enter the USER ID.

-2. Press the “Delete User” Button. (For details, please refer to FAM 90.23 serial command description.doc D.6.)

-3. To erase all users, press the “Erase All” Button. (For details, please refer to FAM 90.23 serial command description.doc D.1.)

5. Toggle the user status

-1. Check for VIP. Uncheck for ordinary user.

-2. Enter the USER ID.

-3. Press the “Change Status” Button. If the input status is same as that of corresponding user, error code 0x47 will be returned.

For details, please refer to FAM 90.23 serial command description.doc C.3.