Description of commands sending from host controller to Ethernet-FAM(ver 80.00)

June 28, 2006.

A. General guideline.

| StartByte | Command Byte | Param1 | Param2 | Error/Flag | CheckSum | StopByte |
| --- | --- | --- | --- | --- | --- | --- |
| 1byte | 1byte | 4byte | 4byte | 1byte | 1byte | 1byte |

1. Start code: 1 byte. Indicates the beginning of a packet. 0x40, corresponding to ‘@.’

2. Command: 1 byte. Refer to the Command Table in a later chapter of this document.

3. Param1: 4 bytes. Indicates user ID or system parameters.

4. Param2: 4 bytes. Indicates second part of user ID or the size of binary data following the command packet such as fingerprint templates or images.

5. Error/Flag: 1 byte. Indicates flag data in the request command sent to the module, and error code in the response command received from the module, respectively.

6. Checksum: 1 byte. Checks the validity of a packet. Checksum is a remainder of the sum of each field, from the Start code to Error/Flag, divided by 256 (0x100).

7. End code: 1 byte. LF (‘\n’, 0x0D). Indicates the end of a packet. Also used as a code indicating the end of a binary data such as fingerprint templates.

COM port parameters:

115200, 8bit, 1 stop bit, no parity, no handshaking

Sample – minutia list from one fingerprint image.

Template – user data in data base (each template consists 1..12 samples)

B. Basic commands for fingerprint registration and recognition.

- Check finger(done)

|  | Code of command | Param1 | Param2 | Flag/Error | Description |
| --- | --- | --- | --- | --- | --- |
| Request from Host | 0x4B | 0x00000000 | 0x00000000 | 0x00 | Check and inform host if there is finger on the scanning window |
| Return to Host | 0x00 | Contrast value | 0x00000000 | 0x40 (if finger is) 0x41(if there isn’t finger ) |  |

It takes 0.1…0.16 second now.

2. Capture fingerprint image (done)

|  | Code of command | Param1 | Param2 | Flag/Error | Description |
| --- | --- | --- | --- | --- | --- |
| Request from Host | 0x49 | 0x00000000 | 0x00000000 | 0x00 | Image of existing on a window finger is stored to FAM’s RAM |
| Return to Host | Dosage Number | Contrast value | Number White pixels | 0x40 (0x41) |  |

Typical time to complete: 0.5 second.

- Process the captured fingerprint image(done)

|  | Code of command | Param1 | Param2 | Flag/Error | Description |
| --- | --- | --- | --- | --- | --- |
| Request from Host | 0x50 | 0x00000000 | 0x00000000 | 0x00 | Process image to sample |
| Return to Host | 0x00 | 0x00000000 | 0x00000000 | 0x40 (0x42,0x43) |  |

Typical time to complete: 0.9 second.

- Find the fingerprint of an ordinary user (1 to 1 matching).(done)

|  | Code of command | Param1 | Param2 | Flag/Error | Description |
| --- | --- | --- | --- | --- | --- |
| Request from Host | 0x52 | Low 4 bytes of User ID | Upper 4 bytes of User ID | 0x00 | Compare current sample with ID template |
| Return to Host | (Result>>2) | 0x00000000 | 0x00000000 | 0x40 (0x4d,0x45) |  |

This function used for compare RamSlotN = 1;

- Find the fingerprint of a VIP user (1 to many matching).(done)

|  | Code of command | Param1 | Param2 | Flag/Error | Description |
| --- | --- | --- | --- | --- | --- |
| Request from Host | 0x52 | 0x00000000 | 0x00000000 | 0x01 | Compare current sample with VIP users templates database |
| Return to Host | (Result>>2) | Low 4 bytes of User ID | Upper 4 bytes of User ID | 0x40 (0x4d,0x45) |  |

This function used for compare RamSlotN = 1;

6. Compare current sample with template in RAM.

|  | Code of command | Param1 | Param2 | Flag/Error | Description |
| --- | --- | --- | --- | --- | --- |
| Request from Host | 0x52 | RamSlotN | 0x00000000 | 0x02 | Compare current sample with template in RAM |
| Return to Host | (Result>>2) | 0x00000000 | 0x00000000 | 0x40 (0x45) |  |

RamSlotN must be in range 0..3.

7. Compare two templates in RAM.

|  | Code of command | Param1 | Param2 | Flag/Error | Description |
| --- | --- | --- | --- | --- | --- |
| Request from Host | 0x52 | RamSlotN_0 | RamSlotN_1 | 0x03 | Compare two templates in RAM |
| Return to Host | 0x00 | 0x00000000 | 0x00000000 | 0x40 (0x45) |  |

RamSlotN_# must be in range 0..3.

Security level is minimal between Global Secure Level and Secure level from RamSlotN_0.

When this function is used, current sample (from function Process image to sample) will be lost.

8. Store the fingerprint template to FAM flash memory.

|  | Code of command | Param1 | Param2 | Flag | Description |
| --- | --- | --- | --- | --- | --- |
| Request from Host | 0x41 | Low 4 bytes of User ID | Upper 4 bytes of User ID | #bXxxxxXXX | Storage template to FAM flash memory |
| Return to Host | 0x00 | Low 4 bytes of User ID | Upper 4 bytes of User ID | 0x40 (0x46) (0x4e) |  |

User ID must be in range 0..0xFFFFFFFFFFFF.

Flag

bit[1:0]	- security level for individual user(0 - minimum,1,2,3 - maximum);

bit[2]		- VIP / ordinary user: if bit[2] ==1 – VIP, bit[2] == 0  ordinary;

bit[3..6]	- reserved for future use;

bit[7]		- if  bit[7] == 0, then template add to database with ID, security level,VIP/ordinary, as set in current command;

bit[7]		- if  bit[7] == 1, then template stored with default parameters (use this bit  equal one only after command “Upload individual template”) and ID, security level,VIP/ordinary,trouble settet in current command ignored;

This function used for source template RamSlotN = 0;

9. Store the sample in FAM RAM.

|  | Code of command | Param1 | Param2 | Flag | Description |
| --- | --- | --- | --- | --- | --- |
| Request from Host | 0x53 | Sample N | 0x00000000 | 0x00 | Store current sample in RAM |
| Return to Host | 0x00 | 0x00000000 | 0x00000000 | 0x40 |  |

Sample N is reserved in range 0..9. The number of samples depends on fingerprint features.

This function used for target RamSlotN = 0;

10. Cancel/Escape.

|  | Code of command | Param1 | Param2 | Flag/Error | Description |
| --- | --- | --- | --- | --- | --- |
| Request from Host | 0x4C | 0x00000000 | 0x00000000 | 0x00 | Escape from current sequence |
| Return to Host | 0x00 | 0x00000000 | 0x00000000 | 0x40 |  |

You cannot interrupt command!

You can interrupt sequence (Add user sequence or Recognize sequence) after any command without use this command.

C. System administration commands.

1. Get hardware and firmware version number.

|  | Code of command | Param1 | Param2 | Flag | Description |
| --- | --- | --- | --- | --- | --- |
| Request from Host | 0x00 | 0x00000000 | 0x00000000 | 0x00 | Get version F/W,H/W |
| Return to Host | 0x00 | F/W low byte, 0x00, F/W high byte, 0x00 | H/W low byte, 0x00, H/W high byte, 0x00 | 0x40 |  |

2. Get free memory space of FAM’s NAND flash memory.

|  | Code of command | Param1 | Param2 | Flag | Description |
| --- | --- | --- | --- | --- | --- |
| Request from Host | 0x4F | 0x00000000 | 0x00000000 | 0x00 | Get free space. |
| Return to Host | 0x00 | Number of free pages (Max=16384) | 0x00000000 | 0x40 |  |

3. Toggle the user status between VIP and ordinary user.

|  | Code of command | Param1 | Param2 | Flag | Description |
| --- | --- | --- | --- | --- | --- |
| Request from Host | 0x47 | Low 4 bytes of User ID | Upper 4 bytes of User ID | #bxxxxXXXX | Toggle the user status between VIP and ordinary user |
| Return to Host | 0x00 | 0x00000000 | 0x00000000 | 0x40 (0x4D) (0x47) |  |

User ID must be in range 0..0xFFFFFFFFFFFF.Flag

bit[1:0]	- security level for individual user(0 - minimum,1,2,3 - maximum);

bit[2]		- VIP / ordinary user: if bit[2] ==1 – VIP, bit[2] == 0  ordinary;

bit[3..7]	- reserved for future use;

4. Set security level for all users.

|  | Code of command | Param1 | Param2 | Flag | Description |
| --- | --- | --- | --- | --- | --- |
| Request from Host | 0x4A | Security level(0..3) | 0x00000000 | 0x00(for get level only) 0x01 (for change level) | Set/get security level for all users. |
| Return to Host | 0x00 | Security level(0..3) | Threshold (100...500) | 0x40 |  |

0 – minimum, 3 – maximum(default).

5. Set security threshold.

|  | Code of command | Param1 | Param2 | Flag | Description |
| --- | --- | --- | --- | --- | --- |
| Request from Host | 0x4e | STH0,STH1 | STH2,STH3 | 0x00(for get thresholds only) 0x01 (for change thresholds) | Set security threshold. |
| Return to Host | 0x00 | STH0,STH1 | STH2,STH3 | 0x40 |  |

STH0..3 – security threshold (0...65535).

D. Data manipulating commands

1. Erase all templates.

|  | Code of command | Param1 | Param2 | Flag | Description |
| --- | --- | --- | --- | --- | --- |
| Request from Host | 0x45 | N/A | N/A | 0x00 | NAND flash is erased and host is informed about number of free 528 bytes pages |
| Return to Host | 0x00 | Number of free pages (Max=16384) | N/A | 0x40 |  |

- Download RAW image (320x480).

|  | Code of command | Param1 | Param2 | Flag | Description |
| --- | --- | --- | --- | --- | --- |
| Request from Host | 0x44 | OffSet | Length | 0x00 | Raw image is sent from FAM to host |
| Return to Host | 0x00 | 0x00000000 | 0x00000000 | 0x40 |  |

If(OffSet == 0)

+ Line0( 320bytes ) + Line1( 320bytes ) +.....(depends of length) ..+ CheckSum  + 0x0D;

3. Download individual template.

|  | Code of command | Param1 | Param2 | Flag | Description |
| --- | --- | --- | --- | --- | --- |
| Request from Host | 0x54 | Low 4 bytes of User ID, (Not uset if Flag==0x01) | Upper 4 bytes of User ID, (Not uset if Flag==0x01) | 0x00 (With ID), 0x01 (Without ID; Download current (raw) template ) | Template with (without) ID is sent from FAM to host |
| Return to Host | 0x00 | 0x00000000 | Length | 0x40, 0x4D, 0x47 |  |

+ Byte0 .....Byte(length-1) ..+ CheckSum1  + 0x0D;

This function used RamSlotN = 1 (If Flag = 0), RamSlotN = 0 (If Flag = 1);

4. Download current sample.

|  | Code of command | Param1 | Param2 | Flag | Description |
| --- | --- | --- | --- | --- | --- |
| Request from Host | 0x4d | 0x00000000 | 0x00000000 | 0x00 0x01 | Send current sample to host |
| Return to Host | 0x00 | 0x00000000 | Length | 0x40 |  |

+ Byte0 + Byte1+..... Byte(Length-1) + CheckSum1 + 0x0D;

5. Upload current sample.

|  | Code of command | Param1 | Param2 | Flag | Description |
| --- | --- | --- | --- | --- | --- |
| Request from Host | 0x4d | 0 | Length | 0x02 0x03 | Receive sample from host |
|  | + Byte0 .....Byte(length-1) ..+ CheckSum1 |  |  |  |  |
| Return to Host | 0x00 | 0x00000000 | 0x00000000 | 0x40, 0x47, 0x49, 0x4A |  |

If (flag==0x02) Length = 664bytes;

If (flag==0x03) Length = 582bytes;

6. Load individual template from Flash to RAM.

|  | Code of command | Param1 | Param2 | Flag | Description |
| --- | --- | --- | --- | --- | --- |
| Request from Host | 0x54 | Low 4 bytes of User ID | Upper 4 bytes of User ID | 0x02 (RamSlotN = 0), 0x03 (RamSlotN = 1), 0x04 (RamSlotN = 2), 0x05 (RamSlotN = 3), | Template with ID is loaded from flash to RAM |
| Return to Host | 0x00 | 0x00000000 | 0x00000000 | 0x40, 0x4D,0x47 |  |

7. Upload individual template.

|  | Code of command | Param1 | Param2 | Flag | Description |
| --- | --- | --- | --- | --- | --- |
| Request from Host | 0x55 | RamSlotN | Length | 0x00 | Template sent from host to FAM |
|  | + Byte0 .....Byte(length-1) ..+ CheckSum1 |  |  |  |  |
| Return to Host | 0x00 | 0x00000000 | 0x00000000 | 0x40, 0x49, 0x4A |  |

RamSlotN must be in range 0..3.

If you will use command Store the fingerprint template to FAM flash memory after this command please set RamSlotN = 0.

8. Erase individual template.

|  | Code of command | Param1 | Param2 | Flag | Description |
| --- | --- | --- | --- | --- | --- |
| Request from Host | 0x48 | Low 4 bytes of User ID | Upper 4 bytes of User ID | 0x00 | Erase individual template |
| Return to Host | 0x00 | 0x00000000 | 0x00000000 | 0x40 (0x4D) |  |

User ID must be in range 0..0xFFFFFFFFFFFFFFFF.

9. Download data from Boot flash.

|  | Code of command | Param1 | Param2 | Flag | Description |
| --- | --- | --- | --- | --- | --- |
| Request from Host | 0x42 | External flash Address | Length | 0x00 | Download data from boot flash. |
| Return to Host | 0x00 | 0x00000000 | 0x00000000 | 0x40 |  |

+ bytes0 + bytes1 +.....(depends of length) ..+ CheckSum1  + 0x0D;

Length must be in range 0x00 ...0xFFFF;

10. Upload data to External Memory Blackfin.

|  | Code of command | Param1 | Param2 | Flag | Description |
| --- | --- | --- | --- | --- | --- |
| Request from Host | 0x0D | Address | Length | 0x00 | Upload data to External RAM |
|  | + Byte0 .....Byte(Length-1) ..+ CheckSum1 |  |  |  |  |
| Return to Host | 0x00 | 0x00000000 | 0x00000000 | 0x40, 0x49, 0x4A |  |

11. Download data from External Memory Blackfin.

|  | Code of command | Param1 | Param2 | Flag | Description |
| --- | --- | --- | --- | --- | --- |
| Request from Host | 0x0F | External flash Address | Length | 0x00 | Download data from External RAM. |
| Return to Host | 0x00 | 0x00000000 | 0x00000000 | 0x40 |  |

+ bytes0 + bytes1 +.....(depends of length) ..+ CheckSum1  + 0x0D;

12. Write firmware from RAM to flash.

|  | Code of command | Param1 | Param2 | Flag | Description |
| --- | --- | --- | --- | --- | --- |
| Request from Host | 0x10 | Length | 0x00000000 | 0x00 | Write firmware |
| Return to Host | 0x00 | 0x00000000 | 0x00000000 | 0x40, 0x47 |  |

13. Change communication baud rate.

|  | Code of command | Param1 | Param2 | Flag | Description |
| --- | --- | --- | --- | --- | --- |
| Request from Host | 0x39 | 0x00000001 – 9600, 0x00000005 – 115200, 0x00000007 – 460800 | 0x00000000 | 0x00 | Change communication rate |
| Return to Host | 0x00 | 0x00000001 – 9600, 0x00000005 – 115200, 0x00000007 – 460800 | 0x00000000 | 0x40 |  |

0 – 4800; 1 - 9600 ; 2 – 19200 ; 3 – 38400 ; 4 - 57600 ; 5 – 115200 ; 6 – 230400 ; 7 – 460800 ;  8 – 921600.

14. Temperature Sensor control.

|  | Code of command | Param1 | Param2 | Flag | Description |
| --- | --- | --- | --- | --- | --- |
| Request from Host | 0x56 | Toti,Thyst | 0x00000000 | #bxxxxxxXX | Set/get sensor data. |
| Return to Host | 0x00 | Toti,Thyst | 0x0000,Temp | 0x40, 0x55 |  |

Toti 	– 16bit signed short (LSByte first);

Thyst 	– 16bit signed short (LSByte first);

Temp 	– 16bit signed short (LSByte first);

Flag

bit[0]		- if (bit[0]==1) then overwrite Toti register;

bit[1]		- if (bit[1]==1) then overwrite Thyst register;

bit[2..7]	- reserved for future use (please set to 0);

15. Software reboot.

|  | Code of command | Param1 | Param2 | Flag | Description |
| --- | --- | --- | --- | --- | --- |
| Request from Host | 0xFF | 0x00000000 | 0x00000000 | 0x00 | Reboot from flash memory. |
| Return to Host | 0x00 | 0x00000000 | 0x00000000 | 0x40 |  |

Reboot start after send response to host.

16. LED/sensor control.

|  | Code of command | Param1 | Param2 | Flag | Description |
| --- | --- | --- | --- | --- | --- |
| Request from Host | 0x11 | 0x000000XX | 0x000000XX | 0x00 | Set/get sensor data. |
| Return to Host | 0x00 | 0x0000000X | 0x00000000 | 0x40, 0x55 |  |

If bits 0..4 Param1 set -> ON corresponding LEDs;
If bits 0..4 Param2 set -> OFF corresponding LEDs;
bit 0 ->> Buzzer
bit 2 ->> Lock ON;
bit 3 ->>LED1;
bit 4 ->>LED2;
return Param1 bit0 ->door sensor0 state;
return Param1 bit1 ->door sensor1 state;

17. Download User list.

|  | Code of command | Param1 | Param2 | Flag | Description |
| --- | --- | --- | --- | --- | --- |
| Request from Host | 0x57 | 0x00000000 | 0x00000000 | 0x01 | Download data from External RAM. |
| Return to Host | 0x00 | 0x00000000 | Length in bytes | 0x40 |  |

+ bytes0 + bytes1 +.....(depends of length) ..+ CheckSum1  + 0x0D;

binary data format: User_IDL(4bytes),User_IDH(4bytes), User_Type(1bytes),3bytes aligner.

User_Type : bit[0..1]-Seq. level,bit[2]-1-VIP/0-Ordinary;

E. Error Codes

| Code | Description |
| --- | --- |
| 0x40 | RESULT_OK |
| 0x41 | RESULT_NO_IMAGE |
| 0x42 | RESULT_BAD_QUALITY |
| 0x43 | RESULT_TOO_LITTLE_POINTS |
| 0x44 | RESULT_EMPTY_BASE |
| 0x45 | RESULT_UNKNOWN_USER |
| 0x46 | RESULT_NO_SPACE |
| 0x47 | RESULT_BAD_ARGUMENT |
| 0x49 | RESULT_CRC_ERROR |
| 0x4A | RESULT_RXD_TIMEOUT |
| 0x4D | RESULT_USER_ID_IS_ABSENT |
| 0x4e | RESULT_USER_ID_IS_USED_ALREADY |
| 0x4f | RESULT_VERY_SIMILAR_SAMPLE |
| 0x55 | RESULT_UNKNOWN_COMMAND |
| 0x57 | RESULT_INVALID_STOP_BYTE |
| 0x58 | RESULT_HARDWARE_ERROR |
| 0x59 | RESULT_BAD_TEST_OBJECT |
| 0x5A | RESULT_BAD_FLASH |
| 0x5B | RESULT_TOO_MANY_VIP |